HarckRF :

- driver=HackRF

- hardware=HackRF One

- clock source=internal

- part id=a000cb3c004b4772

- serial=0000000000000000675c62dc309f6bcf

- version=unknown

BaldeRF:

- driver=bladeRF

- hardware=bladerf2

- fpga_size=40

- fpga_version=0.14.0

- fw_version=2.4.0-git-a3d5c55f

- serial=c1ed7930fbad41d5bdc41cc810266af3

Bandwidth: 10 Mhz

Noise floor Level of Spectrum: -94.70 dbm/10 MHz   (-164.70 dBm/hz)

GNSS signal: -80.60 dbm/10 MHz   (-150.60 dBm/hz)

| Signal Type | Digital gain (dB) | Attenuator | Channel Power (dBm) | PSD (dBm/Hz) | 2d Acc (m) | 3d acc (m) | AGC (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Linear Wide Fast | 0 | 0 | -75.65 | -145.65 |  |  |  |
| Linear Wide Fast | 0 | 20 + Dc blk | -94.01 | -164.01 | 0.77 | 1.20 | 42.9 |
| Linear Wide Fast | 11 | 20 + Dc blk | -89.80 | -159.80 |  |  |  |
| Linear Wide Fast | 17 | 20 + Dc blk | -84.90 | -154.90 | 0.82 | 1.29 | 38.6 |
| Linear Wide Fast | 22 | 20 + Dc blk | -80.05 | -150.05 | 0.90 | 1.44 | 38.6 |
| Linear Wide Fast | 27 | 20 + Dc blk | -75.15 | -145.15 | 0.96 | 1.64 | 34.3 |
| Linear Wide Fast | 32 | 20 + Dc blk | -70.40 | -140.40 | 1.01 | 1.75 | 25.7 |
| Linear Wide Fast | 37 | 20 + Dc blk | -65.05 | -135.10 | 1.35 | 2.68 | 21.4 |
| Linear Wide Fast | 42 | 20 + Dc blk | -60.10 | -130.10 | 2.47 | 4.83 | 12.9 |
| Linear Wide Fast | 47 | 20 + Dc blk | -55.20 | -125.20 | Blind -15 | Blind -15 | 8.6 |
| Linear Narrow | 0 | 0 | -73.31 | -143.31 |  |  |  |
| Linear Narrow | 0 | 23 + Dc blk | -94.00 | -164.00 | 0.77 | 1.20 | 42.9 |
| Linear Narrow | 11 | 23 + Dc blk | -90.15 | -160.15 |  |  |  |
| Linear Narrow | 17 | 23 + Dc blk | -85.30 | -155.30 | 0.79 | 1.41 | 42.9 |
| Linear Narrow | 23 | 23 + Dc blk | -79.75 | -149.75 | 0.82 | 1.43 | 38.6 |
| Linear Narrow | 27 | 23 + Dc blk | -74.80 | -144.80 | 0.86 | 1.43 | 38.6 |
| Linear Narrow | 32 | 23 + Dc blk | -70.30 | -140.30 | 1.01 | 1.66 | 34.3 |
| Linear Narrow | 37 | 23 + Dc blk | -65.20 | -135.20 | 1.27 | 1.99 | 21.4 |
| Linear Narrow | 42 | 23 + Dc blk | -60.02 | -130.02 | 3.08 | 4.37 | 17.1 |
| Linear Narrow | 47 | 23 + Dc blk | -55.15 | -125.15 | Blind | Blind | 8.6 |
| Triangular | 0 | 0 | -75.19 | -145.19 |  |  |  |
| Triangular | 0 | 20 + Dc blk | -93.90 | -163.90 | 0.77 | 1.24 | 38.6 |
| Triangular | 10 | 20 + Dc blk | -90.10 | 160.10 |  |  |  |
| Triangular | 16 | 20 + Dc blk | -85.38 | -155.38 | 0.81 | 1.36 | 38.6 |
| Triangular | 21 | 20 + Dc blk | -80.40 | -150.40 | 0.88 | 1.49 | 38.6 |
| Triangular | 27 | 20 + Dc blk | -74.64 | -144.64 | 0.90 | 1.60 | 34.3 |
| Triangular | 31 | 20 + Dc blk | -70.18 | -140.18 | 1.01 | 1.81 | 25.7 |
| Triangular | 36 | 20 + Dc blk | -65.14 | -135.14 | 1.18 | 2.20 | 17.1 |
| Triangular | 41 | 20 + Dc blk | -59.80 | -129.80 | 1.26 | 2.55 | 12.9 |
| Triangular | 47 | 20 + Dc blk | -54.01 | -124.01 | 1.51 | 2.84 | 8.6 |
| Triangular Wave | 0 | 0 | -76.70 | -146.70 |  |  |  |
| Triangular Wave | 0 | 20 + Dc blk | -94.02 | -164.02 | 0.83 | 1.41 | 38.6 |
| Triangular Wave | 11 | 20 + Dc blk | -90.13 | -160.13 |  |  |  |
| Triangular Wave | 18 | 20 + Dc blk | -85.02 | -155.02 | 0.87 | 1.50 | 38.6 |
| Triangular Wave | 23 | 20 + Dc blk | -79.85 | -149.85 | 0.88 | 1.51 | 38.6 |
| Triangular Wave | 28 | 20 + Dc blk | -74.80 | -144.80 | 0.95 | 1.51 | 34.3 |
| Triangular Wave | 33 | 20 + Dc blk | -69.70 | -139.70 | 1.06 | 1.71 | 25.7 |
| Triangular Wave | 38 | 20 + Dc blk | -64.65 | -134.65 | 1.41 | 2.24 | 17.1 |
| Triangular Wave | 42 | 20 + Dc blk | -60.15 | -130.15 | 2.44 | 3.55 | 12.9 |
| Triangular Wave | 47 | 20 + Dc blk | -55.60 | -125.60 | Blind-5 | Blind-8 | 8.6 |
| Tick | 0 | 0 | -75.72 | -145.72 |  |  |  |
| Tick | 0 | 20 + Dc blk | -94.00 | -164.00 | 0.84 | 1.49 | 42.9 |
| Tick | 11 | 20 + Dc blk | -89.91 | -159.91 |  |  |  |
| Tick | 17 | 20 + Dc blk | -85.10 | -155.10 | 0.88 | 1.51 | 38.6 |
| Tick | 22 | 20 + Dc blk | -80.08 | -150.08 | 0.89 | 1.53 | 38.6 |
| Tick | 27 | 20 + Dc blk | -75.00 | -145.00 | 0.92 | 1.71 | 34.3 |
| Tick | 31 | 20 + Dc blk | -70.43 | -140.43 | 0.93 | 1.71 | 25.7 |
| Tick | 36 | 20 + Dc blk | -65.30 | -135.35 | 1.36 | 2.40 | 21.4 |
| Tick | 41 | 20 + Dc blk | -60.18 | -130.18 | 2.11 | 3.49 | 12.9 |
| Tick | 47 | 20 + Dc blk | -54.52 | -124.52 | Blind-6 | Blind-6 | 8.6 |

- Digital gain is GNU radio slider for transmitted gain.